SenseMinds 360

Technical Walkthrough

Architecture · Data flow · Scenario dry runs · 20 July 2026

Architecture, data flow, and scenario dry runs

A predictive-maintenance platform for six utility machines at Laurus Labs. This document explains how it is built, follows a single sensor reading through every layer of it, and shows the system handling six real scenarios.

Every figure, query result and JSON payload was captured from the running stack. Nothing is illustrative.

*Captured 20 July 2026 · 6 machines · 231 automated tests passing*

# PART I — THE ARCHITECTURE

## 1. The whole system on one page

Figure 1 — The complete platform. Solid arrows carry facts; dashed arrows carry advisory hypotheses.

Read it top to bottom. Sensor data enters at Layer 1, becomes engineering facts at Layer 2, acquires *meaning* at Layer 3, is enriched by machine learning at Layer 4, and is explained in natural language at Layer 5.

The single most important thing in that diagram is the direction of the dashed arrows. Machine learning reads from the deterministic layers and writes only hypotheses back. Nothing downstream treats an ML output as authoritative.

The governing rule of the architecture: deterministic facts are authoritative; machine learning is strictly additive. Delete the entire ML layer and nothing deterministic changes — the platform keeps working, it just stops noticing things nobody wrote a rule for.

### Why this shape

| Decision | Reason |
| --- | --- |
| Rules before ML | A supervised model trained without labelled failures cannot be validated, so its false-positive rate is unmeasurable. An unmeasurable alarm trains operators to ignore the system. |
| ML kept advisory | Anomaly detection catches the unanticipated, but cannot explain itself. Ranking it beneath deterministic rules keeps the benefit without inheriting the unreliability. |
| A knowledge graph in the middle | Relationships (which sensor belongs to which subsystem, which findings triggered which diagnosis) are the questions relational tables answer worst. §7 |
| LLM last, and constrained | Explanation is a presentation concern. The model narrates evidence; it never computes or decides. §9 |

## 2. The domain object everything is built from: a Finding

Before any layer makes sense, one object has to: a Finding is the platform's immutable engineering claim. Every screen, alert, diagnosis and LLM sentence in the system ultimately points at one.

A Finding is:

an observation, never a measurement. "Discharge pressure reached a critical threshold state" is a finding; 280.45 is the evidence beneath it.

immutable and append-only, enforced by a database trigger that rejects UPDATE and DELETE. The record of what the platform believed, and when, cannot be rewritten.

required to carry evidence. Construction without at least one piece of evidence is rejected by the type system.

carrying no workflow state and no priority. Those belong to the systems that consume findings, not to the fact itself.

{ "finding_id": "7a2b317cca6cd33b", "identity_key": "253615721127ab11", "finding_type": "threshold_critical", "origin": "derived", "summary": "discharge_pressure reached a critical threshold state", "detail": "Latest reading 280.45 breached a protection setpoint.", "target_key": "discharge_pressure", "equipment_key": "SC-126", "subsystem_key": "compression", "severity": "critical", "confidence": { "value": 1.0, "rationale": "100.0% coverage (9758 of 9758 readings evaluated)." }, "evidence": [ { "artifact_id": "SC-126__threshold", "description": "current state critical", "observed_value": 280.45 } ], "provenance": { "engine": "findings", "engine_version": "0.1.0", "input_hash": "d6cb92c32cf6af9e", "produced_at": "2026-07-20T06:01:03.495310Z" } }

### Two identifiers, doing different jobs

This distinction runs through the entire system and is worth pausing on:

| Field | Meaning | Changes when |
| --- | --- | --- |
| finding_id | This observation. hash(identity_key, input_hash) | The data changes |
| identity_key | This condition. Stable across observations | Never, for the same condition |

A pressure breach observed at 06:01 and again at 06:02 produces two finding_ids and one identity_key. Alerts, engineer verdicts and the knowledge graph all key on identity_key — because they are about the *condition*, not about one 30-second snapshot of it.

### Three origins, three levels of trust

| Origin | Produced by | Treated as |
| --- | --- | --- |
| derived | An engine, deterministically | Fact |
| diagnosed | The rule engine, from multiple findings | Reasoned conclusion |
| learned | An ML model | Hypothesis — advisory only |

The interface, the alerting policy and the LLM each respect this ranking. Only derived and diagnosed findings at critical severity can raise an alarm.

# PART II — THE LIFE OF ONE SENSOR READING

## 3. From 280.45 to a sentence on a screen

This is the whole platform in one trace. A real reading, real identifiers, real timestamps — the same breach that appears in the alert table in §12.

Figure 2 — One sensor reading, followed through every layer of the system.

Step by step:

06:01:00 — the reading arrives. discharge_pressure = 280.45 on SC-126.

Validation. Timestamp aligned to the 30-second grid, engineering unit confirmed against the sensor catalog, quality flag set to 0 (good). A reading that failed here would be stored but marked, never silently dropped.

Stored. Written to sensor_history.sensor_reading, a TimescaleDB hypertable. The machine's ingest watermark advances to 06:01:00.

Loaded as an `IngestedSeries`. Layer 2 never issues SQL — it receives a typed object containing the validated frame plus the asset manifest.

The threshold engine. 280.45 exceeds SC-126's protection setpoint of 280.0 → state CRITICAL. Six other engines run over the same series in parallel.

A Finding is assembled — 7a2b317cca6cd33b, severity critical, confidence 1.0 with the rationale that 9,758 of 9,758 readings were evaluated, evidence pointing at the threshold engine's artifact.

Projected into the knowledge graph. Two edges are written:

condition:253615721127ab11 --observed_on--> sensor:SC-126:discharge_pressure condition:253615721127ab11 --has_evidence--> artifact:SC-126__threshold

Read as a sentence: *this condition was observed on SC-126's discharge pressure sensor, and the evidence for it is the threshold engine's output.*

The rule engine looks for combinations. Sustained high discharge pressure together with condenser-side degradation and normal suction conditions matches a stored fouling signature → a diagnosed finding, "condenser fouling suspected", carrying triggered_by links to every finding that caused it.

Health cascades up the hierarchy. The sensor-level problem reduces compression-subsystem health, which reduces equipment health. These are two further findings — not separate faults, but the same breach expressed at higher levels of the asset.

The alert policy runs inside the same transaction. Three conditions are newly critical, so three alert rows are written *with* the findings.

06:01:05 — the email. After the transaction commits, the dispatcher groups all three into one report and delivers it. Two seconds from breach to inbox.

The Copilot can now answer questions about it, citing 7a2b317cca6cd33b for anything it says.

The dashboard shows it in plain English: *"Discharge pressure has gone past its safe limit — it is at 280.45 right now."*

Everything after step 6 is downstream of the Finding. That is why §2 introduced it first.

# PART III — THE LAYERS IN DETAIL

## 4. Layer 1 — Data foundation

Readings are validated before storage: missing values, duplicates, timestamp correction, unit conversion, sensor quality.

time ¦ unit ¦ sensor_key ¦ value ¦ quality ¦ source ------------------------+--------+--------------------+--------+---------+---------- 2026-07-20 06:02:00+00 ¦ SC-126 ¦ suction_pressure ¦ 16.39 ¦ 0 ¦ live_csv 2026-07-20 06:02:00+00 ¦ SC-126 ¦ discharge_pressure ¦ 280.05 ¦ 0 ¦ live_csv 2026-07-20 06:02:00+00 ¦ SC-126 ¦ oil_pressure ¦ 185.49 ¦ 0 ¦ live_csv

A sensor catalog maps each machine's raw column headings to canonical keys. This table exists for a specific reason: thresholds are configured against the plant's own column names, while the platform reasons in normalised keys. Without it, a renamed CSV header would silently detach a sensor from its limits.

unit ¦ sensor_key ¦ source_column ¦ ordinal --------+--------------------+--------------------+--------- SC-126 ¦ suction_pressure ¦ Suction Pressure ¦ 0 SC-126 ¦ discharge_pressure ¦ Discharge Pressure ¦ 1 SC-126 ¦ oil_pressure ¦ Oil Pressure ¦ 2

A watermark per machine records the last ingested timestamp, so each cycle processes only new data rather than rescanning history.

Handoff: DbTimeSeriesSource returns an IngestedSeries. Layer 2 is database-unaware.

## 5. Layer 2 — Deterministic analytics

Seven engines run as pure functions. Identical input always produces byte-identical output, which is what makes every conclusion auditable and reproducible.

| Engine | Question it answers |
| --- | --- |
| Quality | Is this reading trustworthy enough to reason from? |
| Statistics | What is normal for this sensor on this machine? |
| Operating state | Running, idle, or stopped? |
| Operating envelope | Inside its designed envelope? |
| Threshold | Has a value crossed a specification or protection setpoint? |
| Operational timeline | How has behaviour evolved over the period? |
| Reliability | Is the sensor itself drifting, stuck, or degrading? |

### 5.1 How health is computed

Health is not an eighth engine measuring something new. It is a composition of what the others already established, which is why it is deterministic and explainable rather than a model output.

Figure 3 — Health composes four existing engine verdicts, then rolls up the asset hierarchy.

Two properties follow from this design:

It is explainable by construction. A reduced health score can always be decomposed into the specific threshold states, envelope excursions and reliability problems that caused it. The platform reports these directly:

*"What is pulling it down: oil system, evaporator, compression."*

It does not depend on ML. A health score computed by a model would be opaque and label-dependent — unsuitable for a number that drives maintenance decisions.

Health rolls up the asset hierarchy: sensor → subsystem → equipment. This is why a single pressure breach produces three findings rather than one.

## 6. The rule engine — turning findings into diagnoses

A threshold engine can tell you that discharge pressure is critical. It cannot tell you *why*. That requires combining several independent findings, which is what the rule engine does.

Figure 4 — Multiple findings combine into a diagnosis that preserves its own reasoning chain.

A rule is a deterministic, explainable statement of engineering knowledge:

IF sustained high discharge pressure breach AND condenser-side degradation AND suction conditions normal THEN condenser fouling suspected

The output is a Finding like any other, but with origin = diagnosed and one additional field — triggered_by, holding the identity_key of every finding that caused it. This is projected into the graph as triggered_by edges, so the reasoning chain is stored, not recomputed: months later it is still possible to ask why the platform concluded fouling on a particular day, and get the exact findings it reasoned from.

Three design constraints on rules:

Rules read findings, never raw sensor values. A rule cannot invent a measurement.

A rule may reference a learned hypothesis only as optional, low-weight corroboration — never as a required condition. Otherwise an unvalidated model could silently become load-bearing.

Rules are versioned. A diagnosis records which rule version produced it.

Current volume: 723 diagnosed findings against 7,663 measured ones.

## 7. Layer 3 — The knowledge graph, and why it exists

The most common question about this layer is why the data is not simply another relational table. The answer is that the platform's most valuable questions are about relationships, and relationships are what tables answer worst.

Figure 5 — The graph stores engineering meaning: hierarchy, governance, evidence, causation and validation.

Consider the questions an engineer actually asks:

| Question | As a graph | As tables |
| --- | --- | --- |
| What else is affected if this sensor is wrong? | Walk has_sensor / observed_on | Multi-way joins |
| Why did the platform diagnose fouling? | Follow triggered_by | Recursive join over a self-referencing table |
| What evidence supports this condition? | Follow has_evidence | Join through an evidence table |
| Which conditions has an engineer confirmed? | Follow validated_by | Another join |
| Show everything connected to SC-126 | One traversal | A query per relationship type |

Each of these is *possible* relationally. The difference is that in a graph they are the same operation — traversal — and adding a new relationship type costs an edge type rather than a schema migration and a new join path.

What the graph stores, and what it does not: it holds equipment, subsystems, sensors, threshold definitions, conditions, evidence references, discovered patterns, learned models and engineer validations. It does not hold sensor values. Measurements live in TimescaleDB; the graph holds what they *mean*.

### 7.1 Why PostgreSQL rather than Neo4j

The graph is two tables, kg_node and kg_edge, behind a repository interface with two implementations (in-memory for tests, PostgreSQL for production).

A dedicated graph database was rejected at current scale for three reasons, the third decisive:

3,234 nodes and 3,649 edges are trivially served by indexed Postgres.

A second engine means a second operational surface, backup story and failure mode.

Cross-database transactions. A finding and its graph projection must commit atomically. Split across two databases, that guarantee is gone — the system could hold a finding whose meaning was never recorded.

The repository interface exists precisely so that migrating to Neo4j, when multi-plant scale justifies it, replaces one class rather than the architecture.

## 8. Layer 4 — Machine learning

Models never see raw readings. A deterministic feature pipeline stands between the database and every model.

Figure 6 — Models consume engineered, reliability-weighted features, never raw ticks.

The reliability weighting step is the one worth noticing: each sensor's contribution is scaled by the reliability engine's own verdict on it, so a drifting or stuck sensor cannot dominate an anomaly score. This is Layer 2 protecting Layer 4 from itself.

Three models run in production:

| Model | Purpose | Emits |
| --- | --- | --- |
| Isolation Forest | "This window is unlike this machine's history" | novelty_elevated |
| Gaussian Mixture | Multivariate operating regimes | operating_regime_discovered |
| Forecasting (backtest-selected) | Lead time before a limit | forecast_threshold_approach |

Isolation Forest was chosen over an autoencoder for the first release because it is inexpensive, has no training instability, suits the modest feature dimensionality, and — decisively — provides per-feature attribution. The platform can report *which* sensors drove a score:

*"Mostly down to: adu2 pressure (lower than usual), cooling water pressure inlet (higher than usual)."*

An autoencoder returns a reconstruction error that explains nothing.

Forecasting will not promote a complex model unless it beats the simple seasonal-naive baseline by a margin under walk-forward backtesting. The selector defaults to the baseline; complexity has to earn its place per sensor.

Phase B is throttled — it looks for slow trends, so it runs on its own slower cadence rather than every 30 seconds.

## 9. Layer 5 — Grounded reasoning

The LLM is a constrained narrator, not a reasoner. It receives structured evidence and must cite it. This is the most safety-critical boundary in the system: it is where a fluent model could invent an engineering claim.

Figure 7 — Every claim must cite a finding. Uncited claims are deleted before the answer is returned.

Four mechanisms make the output trustworthy:

The evidence retriever selects findings, never raw sensor values. The model cannot see a number the engines have not already reasoned about.

The prompt carries absolute rules, including an explicit *never escalate what the engines did not* instruction, and each finding's severity — so the model cannot infer urgency the deterministic layer never asserted.

The citation validator deletes uncited claims before the response leaves the server. This is enforcement, not instruction.

Claims are categorised fact / diagnosis / hypothesis / forecast, so the interface renders an ML hypothesis differently from a measured fact.

A real answer, captured live:

Q: *"What is wrong with this machine right now?"* A: "The machine SC-126 is currently experiencing several issues. The discharge pressure has reached a critical threshold state with a reading of 281.55, which has breached a protection setpoint (ref: bf9973c17bf65bde). The overall health of the equipment is reduced…" — 8 claims, every one carrying a finding id · model: groq

The response also returns an insufficient list: questions the evidence could not answer. That silence is deliberate and is shown to the user rather than filled in.

# PART IV — IMPLEMENTATION

## 10. Data storage

### 10.1 One database, three schemas

A common misreading is that the platform uses several databases. It uses one PostgreSQL 16 instance with the TimescaleDB extension, containing three logical schemas:

| Schema | Purpose | Tables |
| --- | --- | --- |
| sensor_history | Raw measurements, time-series optimised | sensor_reading, unit_sensor, ingest_watermark |
| knowledge | The knowledge graph | kg_node, kg_edge |
| application | Findings, alerts, labels, audit | asset, finding, alert, feedback, engine_run, report, model_registry, rule_version, app_user, role |

Each store resolves its own connection URL, defaulting to a shared one:

SENSEMINDS_DATABASE_URL = postgresql+psycopg://…/senseminds SENSEMINDS_SENSOR_HISTORY_URL = (unset → falls back to DATABASE_URL) SENSEMINDS_KNOWLEDGE_URL = (unset → falls back to DATABASE_URL) SENSEMINDS_APPLICATION_URL = (unset → falls back to DATABASE_URL)

Splitting any store onto its own server later is therefore a configuration change, not a code change.

### 10.2 What TimescaleDB buys

sensor_reading is a hypertable — Postgres transparently partitions it into week-long chunks:

hypertable_name ¦ chunk_name ¦ range_start ¦ range_end -----------------+------------------+------------------------+------------------------ sensor_reading ¦ _hyper_1_1_chunk ¦ 2026-07-09 00:00:00+00 ¦ 2026-07-16 00:00:00+00 sensor_reading ¦ _hyper_1_2_chunk ¦ 2026-07-16 00:00:00+00 ¦ 2026-07-23 00:00:00+00

Queries prune by time — a 6-hour dashboard query touches one chunk, not 750,827 rows.

Compression after 7 days, segmented by (unit, sensor_key).

`time_bucket()` downsampling in the database — the telemetry endpoint returns 90 points without moving raw rows into Python.

Diagnostic note: pg_stat_user_tables reports sensor_reading as 0 rows. That is expected — rows live in the chunks, not the parent table.

## 11. Transaction and failure boundaries

### 11.1 One transaction across two schemas

A single analysis cycle writes findings, graph nodes and edges, alerts, reports, model metadata and the run record. All of it commits or none of it does, inside one unit of work holding one session spanning both application and knowledge.

The system therefore cannot reach a state where a finding exists but its meaning was never recorded, or where an alert was decided but its finding was rolled back.

### 11.2 Idempotency

engine_run carries UNIQUE(unit, input_hash). A run begins with INSERT … ON CONFLICT DO NOTHING RETURNING, so exactly one worker owns a given input; re-analysing the same data is a no-op returning replayed: true.

Verified under concurrency: five simultaneous analyses of one machine produced one run record, one set of findings, one projection.

### 11.3 What is deliberately outside the transaction

Email dispatch. Alert rows commit *with* the findings — the outbox pattern — and the email is sent *after* commit. Three consequences:

SMTP failure can never roll back an analysis.

An alert can never be lost between "detected" and "emailed": the row is already durable.

Failed sends retry on later cycles, up to 5 attempts, then are marked failed with the error preserved and visible in the interface.

ML failures. Pattern learning and forecasting are wrapped in exception handlers. They are advisory; if a model fails, the deterministic analysis still completes.

# PART V — EVIDENCE

## 12. Scenario dry runs

### Scenario 1 — Normal cycle, nothing wrong

A machine reports; three findings are assembled; all three are unchanged from the previous cycle; nothing is written.

The material-change filter compares each finding against the current view — severity, wording, and evidence values within a 2% relative tolerance — and persists only genuine changes.

Why this exists. Findings are append-only and finding_id derives from the input hash, so without the filter every 30-second tick would write a fresh row for a condition that had not changed. This was a real production defect: 4,006 rows accumulated for 25 conditions, roughly 2,532 rows per hour. Four of the six machines now write zero rows per cycle.

The subtlety that made it work: engines embed numbers in prose ("health is reduced (86.8)"), so naive text comparison always differed. The comparator strips digits before comparing wording, then compares values numerically.

### Scenario 2 — Critical breach to inbox in two seconds

The trace in §3, as recorded:

| Time | Event | Persisted to |
| --- | --- | --- |
| 05:57:00 | Pressure crosses the 280 protection setpoint | sensor_reading |
| 05:57:03 | Threshold engine returns CRITICAL; 3 findings assembled | finding |
| 05:57:03 | Graph projection: condition nodes and edges | kg_node, kg_edge |
| 05:57:03 | 3 alert rows written in the same transaction | alert |
| 05:57:05 | One grouped email delivered | status = sent |

### Scenario 3 — Flapping, and the emails deliberately not sent

This occurred unprompted while this document was being written, and is the clearest evidence the escalation policy is doing real work.

SC-126's discharge pressure oscillated across its setpoint:

t ¦ value ----------+-------- 05:58:30 ¦ 278.79 ← below 05:59:00 ¦ 280.74 ← above 05:59:30 ¦ 279.99 ← below 06:00:00 ¦ 282.70 ← above 06:00:30 ¦ 279.82 ← below 06:01:00 ¦ 280.45 ← above 06:01:30 ¦ 281.63 ← above 06:02:00 ¦ 280.05 ← above 06:02:30 ¦ 279.12 ← below

A naive implementation would have emailed on every crossing. What actually happened, for one condition:

at ¦ kind ¦ status ----------+-----------+------------ 05:57:03 ¦ triggered ¦ sent ← engineer notified 05:58:33 ¦ resolved ¦ sent ← told that it cleared 05:59:03 ¦ triggered ¦ suppressed ← re-triggered inside the 15-min cooldown 05:59:33 ¦ resolved ¦ suppressed ← its trigger was never emailed 06:00:03 ¦ triggered ¦ suppressed 06:00:33 ¦ resolved ¦ suppressed 06:01:03 ¦ triggered ¦ suppressed 06:02:34 ¦ resolved ¦ suppressed 06:03:03 ¦ triggered ¦ suppressed

Across all conditions in that window: 6 emails sent, 21 suppressed.

Three deliberate behaviours are visible:

Cooldown — re-triggering within 15 minutes of its own resolution is flapping, not a new incident.

Suppressed is not discarded — every suppressed alert is stored and shown in the interface, so the instability is visible without the inbox filling.

Paired suppression — a resolved whose triggered was suppressed is also suppressed. The system never reports "resolved" for an incident nobody was told about.

### Scenario 4 — The platform declining to raise an alarm

Historically typical operation (P25-P75 = 144.28–152.21) partially overlaps this threshold — some normal operation sits outside the configured limit.

Most readings fall outside a configured limit, yet no protection setpoint was breached and equipment health is normal. The conclusion is that the limit is wrong, not the machine — so the platform raises a configuration-review finding at info severity and sends no alert.

This behaviour separates an engineering platform from a threshold alarm. It also caught an earlier LLM regression, where the model described SC-126 as "not healthy" on the basis of mis-set thresholds; the prompt now carries an explicit never-escalate rule.

### Scenario 5 — Machine learning finding what no rule covers

Summary: Behaviour unlike history in 57 window(s) (hypothesis) Detail: Principal drivers: adu2_pressure (-2.50), cooling_water_pressure_inlet (+1.72), pressure_on_psa_2_tower (-1.45)

Every individual sensor is within limits — no rule fires. The *combination* is unlike anything the machine has done before.

What the platform does not do is alarm. The finding is origin = learned, severity = info, status = hypothesis, and the interface presents it as an early signal, explicitly not a fault.

### Scenario 6 — The learning loop closing

An engineer opens the novelty finding and answers "Was this worth flagging?"

The verdict is stored, keyed on identity_key:

identity ¦ finding ¦ unit ¦ verdict ¦ author --------------+--------------+--------+--------------------+-------- 570ffdfb43fc ¦ 4efff651b2e1 ¦ SC-126 ¦ false_positive ¦ admin 29c13e236b93 ¦ 8a1c02be7791 ¦ SC-126 ¦ expected_behaviour ¦ admin

It is projected into the graph as its own node, linked by validated_by:

node_id ¦ standing ¦ author --------------------------+------------+-------- validation:570ffdfb43fc ¦ rejected ¦ admin validation:29c13e236b93 ¦ explained ¦ admin

GET /feedback/stats reports label readiness honestly:

{ "labelled_conditions": 2, "total_verdicts": 3, "target": 200, "percent_to_target": 1.0, "phase_c_ready": false }

Why keyed on `identity_key`. A condition receives a new finding_id on every observation, so a verdict keyed to the observation would be orphaned within 30 seconds. Keyed to the condition, the label survives later observations and remains valid after the condition clears — which is precisely the training example worth keeping.

## 13. Edge cases tested

### 13.1 Escalation policy — 17 unit tests

| Case | Expected | Result |
| --- | --- | --- |
| New critical condition | triggered, emailed | ✅ |
| Severity warning | no alert at all | ✅ |
| warning → critical | triggered | ✅ |
| Already critical before alerting existed | announced exactly once | ✅ |
| Re-trigger inside cooldown | recorded, suppressed | ✅ |
| Re-trigger after cooldown | emailed normally | ✅ |
| Resolution of a suppressed trigger | also suppressed | ✅ |
| Still critical past 30 min | reminder | ✅ |
| Still critical within 30 min | silent | ✅ |
| Reminders repeat | further reminders | ✅ |
| Severity drops from critical | resolved | ✅ |
| Condition disappears entirely | resolved | ✅ |
| Clears but was never announced | silent | ✅ |
| Already resolved | not resolved twice | ✅ |
| Cascade of criticals | one alert each, one email | ✅ |
| Wording changes while critical | no re-trigger | ✅ |

### 13.2 Feedback loop — 28 end-to-end cases against the live stack

Run against real PostgreSQL and the real HTTP API, not mocks.

| Case | Expected | Result |
| --- | --- | --- |
| Unauthenticated POST | 401 | ✅ |
| Unknown finding | 404 | ✅ |
| Verdict on a derived finding | 400 | ✅ |
| Verdict on a diagnosed finding | 400 | ✅ |
| Invalid verdict value | 422 | ✅ |
| Note longer than 2,000 characters | 422 | ✅ |
| First verdict | 201 | ✅ |
| Author taken from JWT, not request body | admin | ✅ |
| Same verdict clicked four times | 1 row | ✅ |
| Changed verdict | 2 rows, audit trail kept | ✅ |
| Latest verdict wins | newest is current | ✅ |
| UPDATE application.feedback | rejected by trigger | ✅ |
| DELETE FROM application.feedback | rejected by trigger | ✅ |
| Validation node and edge created | present | ✅ |
| Verdict survives a live analysis run | preserved | ✅ |
| Label outlives the observation it was given on | preserved | ✅ |
| Stats count distinct conditions | 2, not 3 | ✅ |
| phase_c_ready honest | false | ✅ |

A measured threshold breach is not a prediction to confirm — "false positive" is meaningless against a reading that genuinely crossed a setpoint. Only learned findings accept a verdict.

### 13.3 The regression this architecture exists to prevent

Graph node properties are replaced wholesale on upsert. The obvious way to record an engineer's verdict — setting status = confirmed on the pattern node — would have been silently erased by the next Phase-B run. Labels would have rotted invisibly, surfacing months later as an empty training set.

So a verdict is stored on its own node, linked by an edge no other projector touches. The test simulates a re-projection overwriting the condition node, then asserts the verdict survives, with an explicit failure message: *"the engineer's verdict was erased by a model re-run."*

### 13.4 Regressions found and fixed during development

| Defect | Consequence | Fix |
| --- | --- | --- |
| Findings re-recorded every cycle | 4,006 rows for 25 conditions | Material-change filter |
| Numbers embedded in prose | Filter never matched | Strip digits before comparing wording |
| Cleared conditions never disappeared | Stale conditions shown forever | Track observed identities per run |
| GET /runs returned 500 | Endpoint dead since D5 | Missing repository added to unit of work |
| nginx cached the API's IP | 502 after every redeploy | Runtime DNS resolution |
| Postgres refused to start after shutdown | Database unusable | Recreate directories the filesystem drops |
| Simulator ran a stale image | Alerting silently absent | Build profile services on deploy |
| Circular import | Worked only by import order | Lazy import in the dispatcher |

# PART VI — MATURITY AND ROADMAP

## 14. Where the platform stands today

| Capability | Status |
| --- | --- |
| Ingestion, validation, time-series storage | Production |
| Seven deterministic engines | Production |
| Findings, evidence, append-only audit | Production |
| Knowledge graph and projection | Production |
| Rule engine and diagnoses | Production |
| Unsupervised pattern learning | Production (advisory) |
| Forecasting with backtest selection | Production (advisory) |
| Grounded LLM with citation enforcement | Production |
| Escalation email with full lifecycle | Production |
| Engineer feedback / label capture | Production |
| Dashboard, 3D twin, Copilot | Production |
| Supervised failure prediction (Phase C) | Not started — blocked on labels |
| OPC-UA / Modbus ingestion | Not started |
| Maintenance and failure-mode graph nodes | Reserved in schema, not populated |

The platform is complete and operating end-to-end for everything that does not require labelled failure history.

## 15. What is not built, and why

Phase C supervised models. Failure probability and remaining useful life require a labelled breakdown history the plant has not yet provided. The interface shows a clearly-marked design preview so the target is reviewable, but nothing computes those numbers. The feedback loop now generates labels for *alarm quality*; true failure prediction still needs the maintenance register.

Protocol ingestion. Data arrives by CSV and REST. OPC-UA and Modbus adapters are required for direct PLC/SCADA connection.

Maintenance and failure-mode graph nodes. The node types are reserved (FAULT_MECHANISM, FAILURE_MODE, MAINTENANCE_ACTION) but nothing populates them — same blocker.

## 16. Roadmap

### Immediate — hardening before internet-facing deployment

Replace the development JWT secret and admin password.

Pin Python dependencies to tested versions; an unpinned rebuild can shift numerical libraries and undermine the reproducibility the platform rests on.

Configure CORS for a split-origin deployment.

Move artifact storage off the container filesystem.

### Near term — validate what already runs

Obtain the plant's existing breakdown register. With historical failure dates it becomes possible to check retrospectively whether the novelty and forecasting signals fired before known failures. This validates the models already in production without training anything, and is the single highest-value input the plant can provide.

### Medium term — close the data loop

OPC-UA / Modbus adapters for direct historian and PLC connection.

Populate maintenance and failure-mode nodes from the work-order system, turning the knowledge graph into a true plant memory.

Accumulate engineer verdicts toward the ~200-label threshold at which an alarm-quality model becomes trainable.

### Longer term — Phase C

Train and *validate* supervised failure classification and remaining-useful- life models — evaluated on held-out time periods, required to beat the deterministic baseline, with an agreed false-positive rate.

Extend to further plants; at that point the knowledge graph and per-store database URLs make extracting services a configuration exercise rather than a rewrite.

The sequencing matters: each step unlocks the next, and none of them requires revisiting the architecture. The platform was built so that Phase C arrives as an addition, not a rebuild.

# APPENDIX — LIVE SYSTEM STATISTICS

Captured 20 July 2026, 06:00–06:05 UTC.

| Metric | Value |
| --- | --- |
| Machines under continuous analysis | 6 |
| Sensor readings stored | 750,827 |
| Findings recorded | 8,503 |
| — measured (derived) | 7,663 |
| — rule-derived (diagnosed) | 723 |
| — ML hypotheses (learned) | 117 |
| Knowledge graph nodes | 3,234 |
| Knowledge graph edges | 3,649 |
| Analysis cycles completed | 6,660 |
| Escalation alerts recorded | 39 |
| Engineer verdicts captured | 4 |
| Automated tests | 231 passing |
| Database migrations | 7 |
| Analysis cadence | 30 seconds |
| Detection-to-email latency | ~2 seconds |

Knowledge graph composition

node_type ¦ count edge_type ¦ count ----------------------+------- ---------------+------- artifact_ref ¦ 2577 has_evidence ¦ 2606 discovered_pattern ¦ 501 discovered_by ¦ 501 sensor ¦ 77 suggests ¦ 388 finding_condition ¦ 45 has_sensor ¦ 77 subsystem ¦ 26 observed_on ¦ 45 threshold_definition ¦ 21 has_subsystem ¦ 26 equipment ¦ 6 governed_by ¦ 21 learned_model ¦ 3 triggered_by ¦ 7 engineer_validation ¦ 2 validated_by ¦ 2

Table row counts

| Schema | Table | Rows |
| --- | --- | --- |
| sensor_history | sensor_reading | 750,827 |
| sensor_history | unit_sensor | 77 |
| sensor_history | ingest_watermark | 6 |
| knowledge | kg_node | 3,234 |
| knowledge | kg_edge | 3,649 |
| application | finding | 8,503 |
| application | engine_run | 6,660 |
| application | report | 6,660 |
| application | alert | 39 |
| application | feedback | 4 |
| application | asset | 6 |
| application | model_registry | 3 |